DCCREG Turbine

Component Calculation Reference

the governing formula, derivation, and dependency for every part in the netlist

Scope.

This document derives, from first principles or cited sources, the calculation for each of the 43 components in the machine netlist (topology_edge_list.csv) — the symmetric voltage-doubler core, the two resonant islands, the distributed-electromagnet reluctance motor, and the resonator tank. It is a paper-study companion to the calculator tools (index.html and the Pyodide synthesizer): every formula below is exactly the one the code evaluates, with the in-code symbol names. Equations are native Word math objects (editable), not images.

Reading conventions.

Each component family gives: the governing equation(s); a table of variables with their value at the established design anchor (r_out 387 mm, V_target 15 kV, V_strike 20 kV, 3000 rpm); how the value is produced in the calculator (which live field feeds it, and what it depends on); and the source — a literature citation [n] (see References) or, for results derived in this project, the derivation plus the in-repo provenance. Tier tags follow the repo: [OC] derivable from standard physics, [IR] a design/empirical choice, [ME] a method.

Firewall.

Pure electrical/mechanical engineering throughout — capacitor electrostatics, LC resonance, magnetics, spark-gap breakdown, field emission. No substrate or DCCREG theory enters any formula.

## The netlist roster (43 components)

The four subsystems and their reference designators (the names KiCad draws and the calculator binds):

Doubler core:  C1, C2 (input varicaps) · Ca1, Cb1 (transfer caps) · SG1, SG2 (rail-return gaps)

Resonant islands:  Cx3, Cx4 (pickup varicaps) · Lx3, Lx4 (resonant inductors) · SG3a1, SG4a1 (load gaps) · SG3b1, SG4b1 (fire gaps) · BS3, BS4 (FE backstops)

Distributed-electromagnet motor:  L_A1–6, L_B1–6 (stator coils) · C_AR1–6, C_BR1–6 (per-coil DC-block caps)

Resonator tank:  L_R1, L_R2 (split coils) · C_R1 (tank cap)

# 0  Foundations — the shared physics

Three relations recur across every subsystem; they are stated once here and referenced below.

### 0.1  Physical constants

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| ε₀ | vacuum permittivity | 8.8541878128×10⁻¹² F/m |
| μ₀ | vacuum permeability | 4π×10⁻⁷ H/m |
| ρ_Cu | annealed-copper resistivity @20 °C | 1.68×10⁻⁸ Ω·m |

### 0.2  Parallel-plate capacitance

Every fixed and variable capacitor in the machine is a parallel-plate (or sectored parallel-plate) structure: two conductive faces of area A separated by a dielectric of relative permittivity εr and thickness d.

C=ε0εrAd

Source / derivation.  Standard electrostatics [1]. The fringing field is neglected (A ≫ d²), consistent with the brief's scope (fringing out of scope).

### 0.3  Charge-sharing (two-capacitor) loss

When a charged capacitor is connected directly to another at a different voltage, the equalisation dissipates a fixed energy independent of the connecting resistance — the “two-capacitor paradox.” It is the tax the doubler pays on each bucket-brigade transfer, and the loss the resonant islands exist to avoid.

Eloss=12C1C2C1+C2(V1−V2)2

Source / derivation.  Classical result [2]; in-repo localisation in sim/brigade_tax_localize.py.

### 0.4  LC resonance, characteristic impedance, quality factor

The islands, the motor coils, and the tank are all LC circuits. For inductance L and capacitance C in series with loss R:

ω0=1LC,f0=ω02π,Z0=LC,Q=Z0R=ω0LR

Source / derivation.  Standard linear circuits [2].

# 1  Doubler core — C1, C2, Ca1, Cb1, SG1, SG2

The heart of the machine is a symmetric (Bennet) voltage doubler: two rotating variable capacitors (C1, C2) are charged at high capacitance and discharged at low capacitance, the rail-return gaps (SG1, SG2) rectify the swing, and the transfer caps (Ca1, Cb1) carry charge between the two halves. Mechanical work modulating C against the trapped charge is pumped up in voltage each cycle.

## 1.1  C1, C2 — the input variable capacitors (sectored varicaps)

Role.  Rotor/stator sectored plates whose overlap — and thus capacitance — varies between C_min (anti-aligned) and C_max (aligned) as the rotor turns. The modulation is the pump's drive.

The aligned (maximum) capacitance is the sectored parallel-plate area over the vertical gap. With N_sec sectors of which n_kept are active over the active band r_in…r_out:

Akept=nkept1Nsecπrout2−rin2

Cmax=ε0Akeptgv

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| r_in, r_out | active-band inner / outer radius | 95 mm / 387 mm |
| N_sec, n_kept | total sectors / active sectors | 12 / 6 |
| g_v | vertical plate gap | 7 mm |
| C_max | aligned capacitance (computed) | ≈ 280 pF |
| C_min | anti-aligned capacitance (incl. stray floor) | 16 pF |

In the calculator.  Cmax_from_geom(r_in, r_out, g_v, n_kept, N_sec) in sim/design_synth.py; the schematic slot sv_C1 / sv_C2 shows C1max_pF live. C1/C2 feed the pump-gain solver (§1.3).

Source / derivation.  Sectored parallel-plate area + §0.2 [1]; the sectored-disc construction follows reference/SectoredDiscCalculator.jsx and docs/brief-blockC1-geometry-to-rotorcap.md [OC].

## 1.2  Ca1, Cb1 — the bucket-brigade transfer capacitors

Role.  A solid annular bus ring on the back of each stator plate forms a fixed capacitor (Mylar dielectric) that carries charge between the doubler halves each stroke.

Ca=ε0εrAringtMylar

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| A_ring | annular bus-ring electrode area | from active-band radii |
| ε_r, t | Mylar permittivity / thickness | ≈ 3.1 / design |
| C_a = C_b | transfer capacitance | 309 pF |

The transfer tax.  Each Ca/Cb hand-off is a charge-sharing equalisation, so it pays the §0.3 loss; summed over the two dominant phase transfers this is 9.79 mJ/fire (≈ 69 % of the cycle tax). This is intrinsic to the ratchet — see §1.3.

In the calculator.  transferCaps(s) in index.html (C = ε0·εr·A/t, solid annulus); sv_Ca1 / sv_Cb1 show Ca_pF live; Ca/Cb set the doubler matrix in §1.3 and the brigade tax in §0.3.

Source / derivation.  §0.2 parallel-plate [1]; the two-cap tax §0.3 [2]; per-transfer localisation in sim/brigade_tax_localize.py (Σ = 19.58 mJ/cycle) [OC].

## 1.3  Pump gain z and net-electrical fraction η

The doubler is solved as a charge-defined capacitor network: from the node voltages V = (v1,v2,v3,v4) the trapped charges are

Q1=(C1+Cpar)v1+Ca(v1−v2),Q4=(C2+Cpar)v4+Cb(v4−v3),…

each stroke re-solves the network at the new capacitances subject to the rail-return rectification, and the steady per-cycle voltage growth is the pump gain z; the fraction of the mechanical input that leaves as net electrical energy is η:

z=|v1|+|v4||v1′|+|v4′||steady,η=medianΔUWmech

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| z | per-cycle pump gain (voltage ratio) | 1.334 |
| η | net-electrical fraction at the 4-node point | 0.386 |
| C_par | parasitic/stray capacitance floor | 20 pF |

Why these values, not larger.  The same rail-return rectification that makes the ladder pump (conduction at the current zero each cycle) is exactly what forbids recovering the transfer tax by over-resonating the core — “the equalisation IS the pump.” So the core fraction is fixed near η = 0.386; recovery is only available on downstream sinks (the islands, §2.2). The machine's operating efficiency is therefore

ηop=Euseful+frecEisland-taxEuseful+Edoubler-tax+Eisland-tax=6.153+frec·4.40720.347≈0.52

In the calculator.  solve_doubler4 / charges_from_voltages / solve_phase in reference/doubler_core.py (the frozen authority, mirrored from index.html); z_eta_Wmech and operating_point in sim/design_synth.py.

Source / derivation.  Symmetric electrostatic voltage doubler — Bennet's doubler [4], analysed in the charge-pump framework of de Queiroz [3]; z is reproduced to 0.002 % by the de Queiroz analytic eigen-matrix (xsim_queiroz_matrix.py) and to a few % by an independent ngspice charge-defined varicap. The efficiency composition is derived and recorded in docs/efficiency-resolution.md [OC/ME].

## 1.4  SG1, SG2 — the rail-return spark gaps

The doubler has no diodes; rectification is by self-breaking spark gaps that hold off until the gap voltage reaches the strike level, then arc and self-quench. The strike voltage is the breakdown of the gap (sphere gap in vacuum/low-pressure cavity):

Vstrike=Ebdgsg(field form),Vbd=Kgsg0.6 (vacuum law)

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| g_sg | spark-gap spacing | 0.5 mm (rail gaps) |
| E_bd | breakdown field (W-Cu spheres) | ≈ 3 kV/mm |
| K | vacuum total-voltage coefficient | 60 (kV·mm⁻⁰·⁶) |
| V_strike | established strike voltage | 20 kV |

In the calculator.  Gaps carry V_strike_kV live (sv_SG1 / sv_SG2); the holdoff threshold sets the doubler rectification in §1.3 and the firing reach in §2.4. V_bd guards insulation invariant I4 in design_synth.py.

Source / derivation.  High-voltage spark-gap breakdown [6]; the vacuum total-voltage law V_bd ∝ g^0.6 is the Cranberg/area form [6], used as design rule K·g^0.6 [IR]. Sphere field 3 kV/mm from the freeze (doc §5) [IR].

# 2  Resonant islands — Cx3/Cx4, Lx3/Lx4, SG3a1/SG4a1, SG3b1/SG4b1, BS3/BS4

Each island is a flying-bucket pickup capacitor (Cx) that is charged from the rail through a load gap (SGa), rings its charge into the receiving bank through a series inductor (Lx) in a half-cycle resonant transfer, fires through a fire gap (SGb) at strike, and is bounded by a field-emission backstop (BS). The series Lx is the efficiency fix: it turns the lossy direct dump (§0.3) into a near-lossless resonant swap.

## 2.1  Cx3, Cx4 — the island pickup varicaps

Role.  Rotating island plates that pick up charge from the rail at high capacitance; the same sectored parallel-plate geometry as C1/C2 (§1.1), sized to the island band.

Cx,max=ε0Aislandgisl

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| A_island | island plate overlap area | from island band |
| g_isl | island plate gap | 4 mm |
| C_x,max | island pickup capacitance | 471 pF |

In the calculator.  Cx_maxMm field feeds sv_Cx3 / sv_Cx4 and the LC ring (§2.2) and the shuttle (§2.4).

Source / derivation.  §0.2 / §1.1 sectored parallel-plate [1] [OC].

## 2.2  Lx3, Lx4 — the series resonant inductors (the half-cycle transfer)

With Lx in series, the island (C_src) discharges into the bank (C_bank) as an underdamped series-RLC ring. The transfer completes in one half-cycle, at the first current zero, where the inductor hands all its energy to the bank. The effective series capacitance, ring impedance, half-period and peak current:

Ceff=CsrcCbankCsrc+Cbank

Z=LxCeff,t1/2=πLxCeff,ipk=ΔVZ=ΔVCeffLx

The energy a direct dump would lose (§0.3) is E_2cap; the resonant ring instead loses only the resistive dissipation over the half-cycle, which vanishes as the ring Q rises — recovering a fraction f_rec of the tax:

E2cap=12CeffΔV2,Eloss=π2QE2cap,frec=1−π2Q

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| L_x | series island inductor | 1 mH |
| C_src / C_bank | island / receiving bank | 471 pF / 2.64 nF |
| ΔV | island-to-bank voltage step | ≈ 5 kV |
| Q = Z/R | ring quality factor | ≈ 729 (R = 2 Ω) |
| t_1/2 | half-cycle transfer time | ≈ 0.8 µs |
| f_rec | recovered-tax fraction | ≈ 0.996 |

In the calculator.  closed_form(...) and the RK4 integrate(...) in reference/island_resonant_core.py; the live tool calls irc.integrate to get f_rec, which sets η_op (§1.3). sv_Lx3 / sv_Lx4 show Lx_mH. t_1/2 must fit the spark-gap conduction window (invariant I10).

Source / derivation.  Standard series-RLC resonant charge transfer [2]; the loss→0 as Q→∞ and f_rec are derived in reference/island_resonant_core.py and cross-checked by an independent ngspice LC ring to <0.6 % (NGSPICE-CONFIRMS, S2). The resonant-transfer principle follows de Queiroz [3] [OC].

## 2.3  SG3a1/SG4a1 (load) and SG3b1/SG4b1 (fire) — the island gaps

Four self-breaking spark gaps clock the island: the load gaps SGa admit charge from the rail to the island; the fire gaps SGb release the island into the cross-coupled bank when the collapsing-capacitance voltage reaches strike. All obey the breakdown of §1.4:

Vstrike=Ebdgsg

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| g_sg | island gap (sphere) | ball d = 12 mm, freeze §5 |
| V_strike | strike voltage | 20 kV |
| V_ceil | insulation ceiling | 21 kV |

In the calculator.  All four carry V_strike_kV live; the fire gaps gate the shuttle reach (§2.4). Gap placement radius r_gap and ball d set the electrode-overlap timing window (firing-geometry, invariants I11/I12).

Source / derivation.  Spark-gap breakdown [6]; gap roles documented in docs/kicad/gap-topology-of-record.md [IR/OC].

## 2.4  BS3, BS4 — the Fowler–Nordheim field-emission backstops

In parallel with the fire gaps, a field-emission backstop bleeds charge softly once the field exceeds an onset (here 0.6·V_strike). The emission current follows the Fowler–Nordheim law (a cold-cathode tunnelling current), and the bled energy is the time integral of V·I over the swing and dwell:

IFN(V)=AV2exp−BV,B=kVstrike

EFE=∫VIFN(V)dt

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| onset | FE turn-on voltage | 0.6·V_strike = 12 kV |
| I_ref | designed leakage at V_strike | 30 µA |
| A, B | FN coefficients (B = k·V_strike) | fit to I_ref, k = 3 |

In the calculator.  fn_coeffs / fn_current / fe_arc_budget in reference/commutator_real_core.py; sv_BS3 / sv_BS4 show 0.6·V_strike. E_FE is a real, perturbable loss (it is the bounded price of holding the gap off).

Source / derivation.  Fowler & Nordheim, cold field electron emission [5]; the budget integral and arc accounting are in reference/commutator_real_core.py [OC for the FN law; IR for the coefficient fit].

## 2.5  Shuttle reach — V*, E_fire, C_fire (how the island fires)

When the finite rail couples its charge onto the larger island it shares to a diluted common voltage V*; the mechanical capacitance collapse then raises that voltage toward strike, where the fire energy and fire-state capacitance are set by the charge delivered:

V*=CrailVrailCrail+Cx,Efire=12QVstrike,Cfire=QVstrike

In the calculator.  shuttle_Wcoll(Cx) wraps the frozen shuttle (island_charging_cosim); design_synth invariant I10 requires V_strike > V* (the collapse must reach strike) and the ring t_1/2 to fit the SG window.

Source / derivation.  Charge-conserving capacitor coupling + collapse [1] [2]; iteration in sim/island_charging_cosim.py [OC].

# 3  Distributed-electromagnet motor — L_A1–6, L_B1–6, C_AR1–6, C_BR1–6

Twelve stator electromagnet coils (two push-pull groups of six, L_A and L_B) spin the rotor as a reluctance motor. Each coil is series-resonated by its own per-coil DC-block capacitor (C_AR/C_BR), which also blocks the DC rail (a coil is a near short at DC). The drive frequency is set by the firing rate.

## 3.1  Drive frequency

PRF=Nev·nrpm60,fdrive=12PRF  (push-pull, alternate strokes)

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| N_ev | commutation events per revolution | design |
| n_rpm | rotor speed | 3000 rpm |
| f_drive | coil drive frequency | ≈ 150–300 Hz |

Source / derivation.  Kinematic definition of pulse-repetition frequency; demMotor(s) in index.html [OC].

## 3.2  L_A1–6, L_B1–6 — the stator coils (reluctance inductance)

Each coil is wound on a C-core with an air gap facing the rotor pole. Its inductance is the magnetic-circuit (reluctance) result for N turns over a pole-face area A_gap and total air gap l_gap, and it is sized so the coil resonates with its block cap at f_drive:

L=μ0N2Agaplgap⟹N=Llgapμ0Agap,L=1ω2Cblock

The coil resistance (copper) and its loaded quality factor:

Rcoil=ρCulwireAcond,Z0=LCblock,Q=Z0Rcoil

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| A_gap | pole-face area | design (cm²) |
| l_gap | total air gap (n_gaps × clearance) | design (mm) |
| L | coil inductance (resonant with C_block) | ≈ 0.64 H |
| N | turns | from L, l_gap, A_gap |

In the calculator.  demMotor(s) in index.html computes L = 1/(ω²C), N = √(L·l_gap/(μ0·A_gap)), Z0, R_coil, Q. sv_L_A* / sv_L_B* show the design inductance (Block-D static — not computed by the live design_synth).

Source / derivation.  Magnetic-circuit / reluctance inductance L = μ0 N² A/l_gap [7]; series resonance §0.4 [2]; conductor resistance and AWG (§3.3, §A) [OC].

## 3.3  C_AR1–6, C_BR1–6 — the per-coil DC-block / resonating capacitors

Each coil's series capacitor performs two jobs: it blocks the DC rail (so the near-short coil does not crowbar it) and it resonates the coil at the drive frequency. Its value is the series-resonance condition, and the AC ampere-turns the branch can drive — the torque-bearing quantity — follow a closed form that is independent of frequency and turns:

Cblock=1ω2L=1(2πfdrive)2L

NI=(Vrating−Vbias)Cblocklgapμ0Agap,Vcap,pk=Vbias+QVripple

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| C_block | per-coil series cap | 440 nF (target) |
| V_rating | cap voltage rating | 20 kV |
| V_bias | DC bias across the cap | design |
| N·I | AC ampere-turns (∝ torque per branch) | closed form |

Energy / safety.  Each cap stores ½·C·V², a lethal kJ-class store on the spinning stator — which drives the per-coil vs per-group topology choice (same total energy either way).

In the calculator.  demMotor(s) §4 in index.html: the cap rating is the binding limit; sv_C_AR* / sv_C_BR* show the 440 nF design value (Block-D static).

Source / derivation.  Series resonance §0.4 [2]; the ampere-turn closed form N·I = (V_rating−V_bias)·√(C·l_gap/(μ0·A_gap)) is derived in index.html demMotor / the Block-D brief from L = μ0 N²A/l_gap and Z0 = √(L/C) [OC/ME].

# 4  Resonator tank — L_R1, L_R2, C_R1

The output stage is an LC tank: two conical coils in series-aiding (L_R1, L_R2) resonate with a through-mica inter-electrode capacitor (C_R1), pulse-excited by the firing rate. It is built from circular-loop inductance, parallel-plate capacitance, and the LC relations of §0.4.

## 4.1  C_R1 — the through-septum tank capacitor

CR=ε0εdAalignhdisc

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| A_align | aligned rotor-face area through the disc | full rotor face |
| ε_d, h_disc | mica/garolite permittivity / disc thickness | design |
| C_R | tank capacitance | 789 pF |

In the calculator.  resonatorCore(s) in index.html: C_R = ε0·εd·A_align/h_disc; sv_C_R1 shows C_R_pF live. C_R sets f0 and Z0 with L (below).

Source / derivation.  §0.2 parallel-plate through the septum dielectric [1] [OC].

## 4.2  L_R1, L_R2 — the split conical coils (loop inductance)

The tank inductance is summed over the winding's circular loops: each loop contributes its high-frequency self-inductance, and every pair contributes Maxwell's mutual inductance of two coaxial loops (via the complete elliptic integrals K, E). Series-aiding, the two cones add with their mutual.

Lself=μ0rln8ra−2

k2=4r1r2(r1+r2)2+d2,M=μ0r1r22k−kK(k)−2kE(k)

L=iLself,i+2i<jMij

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| r, a | loop radius / wire radius | from cone geometry |
| d | axial spacing between two loops | winding pitch |
| L | total tank inductance (both cones) | ≈ 79 µH (≈ 39.5 µH each) |

In the calculator.  Lself(r,a), Mloop(r1,r2,d) via ellipKE, and L = Σself + 2ΣΣM in index.html resonatorCore; sv_L_R1 / sv_L_R2 show the per-coil design inductance (≈ 39.5 µH).

Source / derivation.  HF self-inductance of a circular loop, L = μ0 r(ln(8r/a) − 2) [8]; Maxwell's mutual inductance of coaxial circular loops via complete elliptic integrals K, E [8] [OC].

## 4.3  Tank resonance, impedance, Q, and the damped drive

With the total L and C_tot = C_R + C_self + stray, the tank resonance, characteristic impedance and copper-limited Q (through the skin-effect AC resistance) are:

f0=12πLCtot,Z0=LCtot,Q=ω0LRac

δ=ρCuπf0μ0,Rac=ρCulwireπdODδ,fd=f01−14Q2

| symbol | meaning | value at the established anchor |
| --- | --- | --- |
| C_tot | tank C incl. self-capacitance + stray | ≈ 789 pF + … |
| f_0 | tank resonant frequency | 637 kHz |
| δ | copper skin depth at f0 | ≈ 80 µm |
| f_d | damped natural frequency | ≈ f0 |

The single-layer self-capacitance of the coil uses the Medhurst estimate (D, H in cm):

Cself=D0.1126HD+0.08+0.27H/D

In the calculator.  resonatorCore(s) in index.html: f0, Z0, Q, skin depth δ, Rac, fd, and the Medhurst Cself; the badge f0/Z0 are surfaced live in Block-R.

Source / derivation.  LC resonance and skin effect §0.4 [2]; Medhurst single-layer self-capacitance [9] [OC/IR].

# 5  How the components depend on each other (the calculator chain)

The calculators are producer/consumer: geometry produces capacitances, the frozen doubler consumes them to produce z/η, the islands recover part of the tax, and a battery of invariants couples the whole. The central dependencies:

Geometry (r_in, r_out, g_v, N_sec)  → C1/C2 C_max (§1.1) and Cx (§2.1)

C1, C2, Ca, Cb, C_par  → pump gain z and η via the doubler network (§1.3)

Cx, L_x, C_bank, ΔV, R  → resonant transfer t_1/2, i_pk, f_rec (§2.2) → η_op (§1.3)

V_strike, g_sg, ball d  → rail/island rectification (§1.4, §2.3) and shuttle reach V* (§2.5)

f_drive, C_block  → coil L (§3.2) → turns N, Z0, Q, N·I (§3.2–3.3)

L (loops), C_R, C_self  → tank f0, Z0, Q (§4.3)

The invariants that bind them (design_synth.py I1–I13).

I2 frozen-solver authority (z, η, W_coll come only from the solvers) · I3 z within the validated band [1.20, 1.45] · I4 insulate-first (gap V_bd > V_target, septum holds) · I5 tax managed (η ≥ 0.15) · I6 parasitic floor (C_par ≥ 20 pF) · I7 motor matched (output ≤ pump net; f_res = PRF) · I9 mechanical (rim speed < 200 m/s) · I10 shuttle integrity (V_strike < ceiling, V* reaches strike, ring t_1/2 fits the SG window) · I11/I12 firing geometry (electrode-overlap window, cross-fire margin) · I13 island recovery. A design is feasible only inside the region all of these allow; the binding one is what limits it.

Producer/consumer discipline.

The geometry/motor/resonator blocks are producers: they compute capacitances/inductances and feed them in, but never write the rotor-cap fields and never call the doubler solver. The frozen doubler (reference/doubler_core.py) is the sole consumer/authority on z and η. This is what keeps the flexible dimensions from quietly overriding a derivation.

# References

[1] D. J. Griffiths, Introduction to Electrodynamics, 4th ed., Cambridge Univ. Press, 2017 — parallel-plate capacitance, charge-conserving coupling.

[2] C. K. Alexander & M. N. O. Sadiku, Fundamentals of Electric Circuits, 6th ed., McGraw-Hill, 2017 — LC resonance, characteristic impedance, quality factor, skin effect, the two-capacitor charge-sharing loss.

[3] A. C. M. de Queiroz, “Electrostatic vibration energy harvesters with separated charge injectors / generalized symmetric charge-pump multipliers,” IEEE — the charge-defined varicap doubler analysis used as the analytic witness for z (eigen-matrix; xsim_queiroz_matrix.py).

[4] Abraham Bennet, “Description of a new electrical doubler,” Phil. Trans. R. Soc. 77, 288 (1787) — the symmetric electrostatic voltage doubler.

[5] R. H. Fowler & L. W. Nordheim, “Electron emission in intense electric fields,” Proc. R. Soc. Lond. A 119, 173–181 (1928) — the cold field-emission (tunnelling) current law I ∝ V² exp(−B/V).

[6] M. S. Naidu & V. Kamaraju, High Voltage Engineering, 5th ed., McGraw-Hill, 2013 — spark-gap and sphere-gap breakdown; the vacuum total-voltage breakdown law V_bd ∝ d^0.6 (Cranberg/area effect).

[7] A. E. Fitzgerald, C. Kingsley, S. D. Umans, Electric Machinery, 6th ed., McGraw-Hill, 2003 — magnetic-circuit / reluctance inductance L = μ0 N² A / l_gap.

[8] F. W. Grover, Inductance Calculations: Working Formulas and Tables, Dover, 1946 — Maxwell's mutual inductance of coaxial circular loops (complete elliptic integrals) and the high-frequency self-inductance of a circular loop.

[9] R. G. Medhurst, “H.F. resistance and self-capacitance of single-layer solenoids,” Wireless Engineer 24, 35–43 & 80–92 (1947) — the single-layer self-capacitance estimate.

[R] In-repo derivations and validations (provenance for the project-specific composite results): reference/doubler_core.py, reference/island_resonant_core.py, reference/commutator_real_core.py, sim/design_synth.py, index.html (demMotor / resonatorCore / transferCaps), docs/efficiency-resolution.md, and the CHANGELOG verdict trail (NGSPICE-CONFIRMS, RESONANT-TRANSFER-MODELED, EFFICIENCY-RESOLVED).

Generated by tools/gen_calc_reference.py — re-run to regenerate from the calculator sources. Equations are native Word math (OMML).